CS3570 Introduction to Multimedia Technology

Homework #3

Due: 11:59pm, May 16, 2012

- Create your own FIR filters to filter audio signal:

With the ideal convolution impulse response functions in Table 5.2 and the windowing functions in Table 5.3, we can easily design a filter according to algorithm 5.1. In this homework, you need to design different filters and test their effects.

- You can modify the two given m-files “hw3.m” and “myFilter.m” for this homework. Some steps are already done in the file for you, but you can write your own files if you want.

- “hw3_mix.wav” consists of 3 songs, you need to apply your filters to separate them from the audio file.

- Discuss how you determine the filters for this homework problem.

- Store the filtered audio files, and plot

- The spectrum of the input signal

- The spectrums of the output signals

- The spectrums of the filters

- The shapes of the filters (time domain)

- Apply filtering in time domain rather than frequency domain, and matlab built-in function “conv” is not allowed to use in this homework. You need to implement 1-D convolution by yourself.

- Only one windowing function is required, but do not use rectangular windowing function.

- You just have to implement the filters you need.

- You have to upload two m-files and three filtered audio files. Show the plots in your report.